AMD Xilinx kria(k26)

Xilinx has been acquired by amd.

It is basically a system on module, which performs dedicated tasks. Som’s are generally an embedded computer packaged into a single module. They are part of an electrical or mechanical system of a device.

It includes:

RAM

ROM

Storage

Power management

Processor

SOM are never meant to be used alone, they are plugged into a carrier board using connectors. SOMs are the core computing power. Carrier boards provide the necessary inputs for the som, it varies based on the application we are working on.

Soms are used where u need more computing power than a microcontroller and don’t want design a computer from scratch.

Soms(like kria) run full-fledged operating systems like linux on them. They also have powerful multi core processors like amd cortex a series. They are used for ai inference video processing.

Jetson nano, orion are similar soms like kria, raspberry is a single board computer, som and carrier board combined into one.

Xilinx is a fpga  based som. Fpga don’t have particular function but we program it at the hardware level to run custom, ultrafast , low latency processing pipelines.

Comparision between all three som’s:

Pynq is a software framework , (python on zynq) it lets you program the fpga using python.

While kria is a hardware component, u can run pynq on kria. On the other hand rio is a certified industrial controller which can only be programmed through labview.

Kria specs:

256k logic cells , more than pynq z2 and ni rio

Quad core processor

Zynq ultrascale+ soc

Memory 4gb ddr ram, more than pynq z2 and ni

Dsp slices - 1,248

Storage 16gb emmc and microsd

Less costlier than ni but more costlier than pynq albeit marginally.

Conclusion:

Kria can be used since its more powerful and due to its versality. It is easily customizable and we can even run pynq on it to make the programming easier.